12-bit, 1350x oversampling

Sigma-Delta A/D Converter

April 2004, Version 1.0 	SSG UK - Preliminary Information	Project Specification

## Features

- ADC function integrated in the FPGA

- Minimal need for external components : RC + signal conditioning

- Avalon interface (integrated into SOPC Builder)

- Input signal bandwidth : 4kHz

- Input voltage range : 0V to 3.3V

- 12-bit data output

- First Order Sigma-Delta Modulator

- Two-Stage Decimation Filter

## Applications

- DC and low-bandwidth (up to 4kHz) measurement

## General Description

The conversion process is based on a sigma-delta modulator that processes the analogue input and generates a train of pulses at a very high sample (oversampling). These pulses are then filtered and decimated so the converted signal can be described by a 12-bit word generated at the desired sample rate. The model diagram for a 1-channel Sigma-Delta ADC is shown in Figure 1.

+

∫

1-bit

ADC

1-bit DAC

Analog Input

Modulated Output

+

-

Converted Signal

Digital Filtering and Decimation

Digital Domain

Analogue Domain

Figure 1: First Order Sigma-Delta ADC Model

## Functional Description

The connection diagram for the Avalon ADC is shown below in Figure 2.

Avalon_ADC

ad_inp

Pin0

Slave

interface

feedbackj

Pin1

reg_0

inv_0

Figure 2- Connection diagram

The possible configuration alternatives are shown below in Table 1:

| Modulator Architecture | Pin0 | Pin1 | inv_0 | reg_0 address |
| --- | --- | --- | --- | --- |
| Internal comparator / Topology 1 | LVDS input pin (signal ad_inp must be assigned to the positive input) | LVTTL output | Bypassed (Pin1 = feedback) | 0x0 (12 LSBs) |
| Internal comparator / Topology 2 | LVDS input pin (signal ad_inp must be assigned to the positive input) | LVTTL output | Should be included at the top level (Pin1 = !feedback) | 0x0 (12 LSBs) |
| External comparator | LVTTL input | LVTTL output | Bypassed (Pin1 = feedback) | 0x0 (12 LSBs) |

Table 1 – Configuration options

The A/D Converter’s modulator and filtering stages can be represented by the block diagram in Figure 3.

OFF-CHIP

##### ON-CHIP

Control

rst_n

ad_ready

sysclk

LVDS input buffer

CIC Filter

Low-Pass

int_clk

+

ad_inp

ad_inp(n)

DFF

(x1350)

Equalizer

ad_out[11:0]

C

R

feedback

Sigma-Delta Modulator

(Topology 1, internal comparator)

Clock domain

sysclk

Decimation and Filtering

Clock domain

sysclk

Roll-off Compensation and Filtering

Clock domain

int_clk = (sysclk/1350)*15

Figure 3- Block diagram

The analogue input is connected to the positive input of an LVDS input buffer (to be assigned to an I/O bank powered with 3.3V) in the case of using the internal comparator. The output of the comparator will then be registered before it is sent back to the integrator and, at the same time, to the first stage of the filtering process.

The CIC block will filter the incoming signals and generate a valid 12-bit output at a rate determined by sysclk/1350. This signal will then be sent to a low-pass filter.

The low-pass filter has two purposes: provide compensation for the strong roll-off of the CIC filter so the filter response is flat within the signal bandwidth, and offer high attenuation for frequencies higher than half the output sample rate. Because of its serial architecture it needs to be clocked at 15 times the sampling frequency (sysclk/1350).

The low-pass filter output is then sent to reg_0 (sampled using sysclk as clock and ad_ready as enable).

## Conversion Time and Output Sampling Rate

The conversion time (or group delay) is the time it takes for the digital output (ad_out) to reflect any changes that occurred in the analogue input (ad_inp). This delay is a function of the filters used and the system clock (sysclk).

Output sampling rate is the system clock frequency (sysclk) divided by the decimation factor included in the CIC filter stage (=1350 for this implementation). This means that every 1350 cycles of sysclk a valid value is sampled into reg_0.

Although the default value for sysclk in the included Quartus project is 27MHz, other frequencies can be used. Restrictions in the Sigma-Delta modulator used will limit the system clock to 50MHz. The relationship between sysclk and conversion time/output sampling rate is shown below in Table 2.

| System Clock (sysclk) | Sampling Rate (sysclk/1350) | CIC Conversion Time (5*1350/sysclk) | LPE Conversion Time (11*1350/sysclk) | Total Conversion Time (worst case) (CIC+2*LPE) |
| --- | --- | --- | --- | --- |
| 27MHz | 20kHz | 250µs | 550µs | 1.350ms |
| 50MHz | ~37kHz | 135µs | 297µs | 0.729ms |

Table 2 – System Clock x Conversion Time

## ADC Gain

After the decimation, the filtering and the bus truncation, the maximum value (modulator output always high) that can appear on ad_out is 0xDC3 (3523 in decimal) and not 0xFFF (4095 in decimal). This is due to the system gain and should be taken in account when compensating for offset and gain errors.

| 101 Innovation Drive San Jose, CA 95134 (408) 544-7000 http://www.altera.com | Copyright  2004 Altera Corporation. Altera, FLEX, MAX+PLUS II, MegaWizard, and AMPP are trademarks and/or service marks of Altera Corporation in the United States and other countries. Other brands or products are trademarks of their respective holders. The specifications contained herein are subject to change without notice. Altera assumes no responsibility or liability arising out of the application or use of any information, product, or service described herein except as expressly agreed to in writing by Altera Corporation. Altera customers are advised to obtain the latest version of device specifications before relying on any published information and before placing orders for products or services. All rights reserved. |
| --- | --- |